# Series Circuits — Student Notes

Unit: Current Electricity — Lesson 03 Name: _______________________ Date: __________ Period: ____

## Learning Targets

By the end of today’s 42 minutes I can:

- Identify a series circuit and explain why the current is the same everywhere in it.

- Compute the equivalent resistance in series (R_total = R₁ + R₂ + …) and find the current.

- Explain why the voltage drops add up to the source voltage.

## Key Vocabulary

- series circuit — _______________________________________________

- equivalent resistance — _______________________________________________

- current — _______________________________________________

## Guided Notes

A series circuit has a single __________ for charge. Because there is only one path:

- The current is the __________ at every point in the loop.

- The total (equivalent) resistance is the __________ of the resistors: > R_total = R₁ + R₂ + R₃ + …

- The voltage drops across the resistors __________ up to the source voltage.

- One break anywhere stops the current __________.

Schematic of a series circuit: one battery drives a single current through three resistors in a row; the current is the same everywhere.

Finding the current. Once you have R_total, use Ohm’s Law: > I = V ÷ R_total (this same I flows through every resistor)

Voltage drops add to the source:

Bar chart of voltage drops in a series circuit summing to the source voltage.

Reference: V = I·R · series R_total = R₁ + R₂ + … · power P = V·I = I²·R

## Worked Example

Problem. A 20 V battery is in series with R₁ = 2 Ω, R₂ = 3 Ω, and R₃ = 5 Ω. (a) Find R_total. (b) Find the current. (c) Find the voltage drop across R₃.

Solution. 1. R_total = R₁ + R₂ + R₃ = 2 + 3 + 5 = __________ Ω. 2. I = V ÷ R_total = 20 ÷ 10 = __________ A (this current flows through every resistor). 3. Voltage across R₃: V₃ = I·R₃ = 2 × 5 = __________ V. (Check: drops are 4 V + 6 V + 10 V = __________ V = the source.)

## Summary

Finish the sentence (Because / But / So):

In a series circuit the current is the same everywhere because __________________________________________, but __________________________________________, so __________________________________________.