# Series Circuits — Answer Key

## Cover

Unit: Current Electricity — Lesson 03: Series Circuits East Meadow Schools × Valley Stream Central HSD Strategy chip: ACTIVE LEARNING

## Make-It-Make-Sense Answers

- R_total = 1 + 2 + 3 = 6 Ω; I = V ÷ R_total = 12 ÷ 6 = 2 A (everywhere). · Rubric: correct sum AND I = V/R_total. · NYSSLS: HS-PS3-6

- V across 3 Ω = I·R = 2 × 3 = 6 V; V across 5 Ω = 2 × 5 = 10 V. · Rubric: uses the common 2 A current with V = I·R for each. · NYSSLS: HS-PS3-6

- The current goes down, because adding a series resistor raises R_total and I = V ÷ R_total. · Rubric: “down” AND links to larger R_total. · NYSSLS: HS-PS3-6

## Exit Ticket Answer

- R_total = 4 Ω + 8 Ω = 12 Ω.

- I = V ÷ R_total = 24 ÷ 12 = 2 A.

- Across the 8 Ω: V₈ = I·R = 2 × 8 = 16 V. Across the 4 Ω: V₄ = 2 × 4 = 8 V. Check: 16 + 8 = 24 V = the source. ✓

Tolerance/rubric: (a) 12 Ω; (b) 2 A; (c) 16 V with a correct sum-check to 24 V. 1 point each. NYSSLS: HS-PS3-6.

## Closing Reflection (rubric)

Formative, not graded. Award full credit for naming a real prediction that was disproved (most common: “current gets used up / shrinks at each bulb”) and what changed their mind (identical ammeter readings).